华为技术有限公司

- 2015年06月

- 版权所有 侵权必究

修订记录

| 日期 | 版本号 | 作者 | 检视人 | 主要修订内容 |
| --- | --- | --- | --- | --- |
| 2015.6.15 | V1.0 |  |  |  |

目  录

1 华为控制器简介	4

1.1 华为敏捷网络产品介绍	4

2 业务开发指南	5

2.1 知识能力准备	5

2.2 控制器GUI与RESTCONF工具	5

2.2.1 控制器GUI	6

2.2.2 RESTCONF tools	9

2.2.2.1 RESTCONF工具介绍	9

2.2.2.2 chrome Postman plug-in	9

2.3 OpenFlow	10

2.3.1 OpenFlow简介	10

2.3.2 OpenFlow拓扑	11

2.3.3 OpenFlow编程概述	18

2.2.3.1 L2 Flow编程接口与示例	18

2.2.3.2 L3 Flow编程接口与示例	21

2.3.4 OpenFlow路径编程	23

2.3.5 OpenFlow统计信息	23

2.3.5.1单条流统计信息接口与示例	24

2.3.5.2 聚合流统计信息接口与示例	26

2.3.5.3 表统计信息接口与示例	27

2.3.5.4 端口描述和统计信息接口与示例	28

2.3.5.5 节点描述接口与示例	31

2.3.6 Host Tracker	43

3 缩略语	44

# 华为控制器简介

## 华为敏捷网络产品介绍

华为敏捷控制器（Agile Controller）是华为面向企业市场发布的下一代网络解决方案敏捷网络的核心部件，全面覆盖敏捷园区、敏捷分支、敏捷数据中心等各种应用场景。

华为敏捷控制器通过将网络设备控制面与数据面分离开来，从而实现了网络流量的灵活控制，为核心网络及应用的创新提供了良好的平台。网络业务可通过SDN控制器的编程，快速满足客户灵活多变的需求，可大量简化网络策略控制。

华为敏捷网络解决方案包括转发层、控制层和应用层，同时对于这三层，可多层次对外开放编程。

转发层，也就是设备层，华为全系列设备都采用相同的操作系统VRP。

控制层是华为自主研发的SDN控制器，支持对传统网络的管理，及对传统网络与SDN网络的融合管理。

应用层：在这一层华为将打造一个SDN生态圈，联合合作伙伴来共同开发SDN应用来满足最终用户各种各样的需求。

# 业务开发指南

## 知识能力准备

开发者除了需具备基本通信网络系统、网络管理和编程的背景外，建议对SDN接口协议相关知识进行必要的了解，以下信息可以参考。

OpenFlow，请参考：

https://www.opennetworking.org/component/content/article/42-sdn-resources/2046-technical-resources.

RESTCONF protocol，请参考：

http://tools.ietf.org/id/draft-bierman-netconf-restconf-00.xml；

https://wiki.opendaylight.org/view/OpenDaylight_Controller:MD-SAL:MD-SAL_Document_Review:RESTCONF

OpenFlow YANG model，请参考：

http://www.rfc-editor.org/rfc/rfc6020.txt

https://git.opendaylight.org/gerrit/gitweb?p=controller.git;f=opendaylight/md-sal/model/model-flow-base/src/main/yang/opendaylight-flowtypes.yang;a=blob.

## 控制器GUI与RESTCONF工具

控制器提供图形化用户接口(GUI)。GUI接口通过REST APIs和控制器交互，并可显示网络拓扑、节点等信息。

控制器提供如下的GUI接口：

- 拓扑；

- 节点

- YANG UI；

除了通过控制器自带的GUI接口，还可以通过其它RESTCONF工具调用控制器的编程接口，如Postman chrome和cURL utility。

### 控制器GUI

登陆到控制器GUI

按照以下步骤即可登陆到控制GUI：

1 打开Chrome浏览器，输入登陆URL http:// <controller-ip>:8181/dlux/index.html#。<controller-ip>即安装控制器的计算机的IP地址；

2 作为admin登陆(ID/password：admin/admin)；

3 点击“login”按钮。

注意：IE浏览器和控制器不兼容，此处需要使用Google Chrome浏览器。

查看网络拓扑

拓扑查看以图形化的方式展示网络拓扑。

控制器根据所有连接到网络的交换机计算网络拓扑，然后控制器GUI根据收集的拓扑信息通过图像化的方式呈现。

该控制器GUI界面并不能提供增加或者更新拓扑信息的能力。

按照以下步骤即可查看网络拓扑：

1 选择“Topology”按钮；

可以看到图形化的网络拓扑。

2 将光标聚焦到主机、交换机或者链路，可以查看相应资源和目的端口；

3通过向上或者向下滚动鼠标轮，可以放大或者缩小网络拓扑图。

查看网络统计信息

GUI页面 “Nodes”按钮提供可以一种查看网络中交换机节点信息和端口统计信息的方式。

1 选择“Nodes”按钮，即可看到一个关于所有节点、节点端口及统计信息的列表。

2 在“Search Nodes”输入框，输入完整的node id，可以看到该节点；输入不完整的node id，则会搜索到匹配的节点；

3 “Node Connectors”列中数字表示交换机的端口数。点击“Node Connectors”的数值，可以看到端口详细信息，如端口ID、端口名称、交换机的端口数、MAC地址等。

4 点击 “statistics”列中的“Flows”，可以看到特定节点的流表统计信息，流表统计信息包括表ID、active flows、packet matc等；

5点击 “statistics”列中的 “Node Connectors”，可以查看节点端口的统计信息。

YANG UI使用

YANG模型针对网络资源定义了数据模型。控制器提供了通过标准REST API方法，如GET、PUT、POST和DELETE，访问网络资源的手段。资源可以是通过XML或者JSON格式呈现的数据或者远程过程调用(remote procedure call ,RPC)。

以下以目录资源(opendaylight-inventory)为例讲述YANG UI使用资源的过程。

选择“YANG UI”按钮，即可使用YANG UI。可以执行以下操作:

1 点击“+”按钮，展开opendaylight-inventory；

打开的页面显示数据结构的第一层：

• config —可配置元素(可读写)；

• operational —运行元素(只读)，即反映当前状态的信息

2 选择并展开config元素

可以看到第一层可配置的资源，即根节点Nodes；

3 点击“Nodes”,在列表的底部，页面会显示元素的xpath，同时提供了可以应用于元素的REST API方法；

4 展开Node {id}，可以看到“{id}”。“{id}”是一个可以配置的参数，对应xpath中的输入框。为配置元素，需要在输入框中输入适当的值。

GET和DELETE方法不需要请求参数体，但是POST和PUT操作需要相应的请求参数体。每一个可配置资源都有特定的参数模板。

5 点击API树后面的“Show Preview”,可以显示发给控制器的请求。填入数据，YANG UI会显示请求描述的文本。

将请求方法发给控制器后，即触发REST请求。在收到“Request sent successfully”之后，点击“Show Preview”可以看到控制器对请求消息的响应。

按照上述步骤，可以配置YANG模型结构下的任何资源。

YANG UI下配置资源列表

YANG UI以树形结构显示，包括列表资源，树形结构可以展开或者折叠。

通过YANG UI，可以执行如下配置动作：

1 点击列表名称后的 “+”按钮，可以添加元素。添加元素后，可以看到以该元素名称标识的的按钮以及关键值；

2点击列表元素之后的“X”按钮，可以移除该列表元素；

3 点击元素关联的按钮，可以选中元素。只有在列表中包括至少一个元素时才会显示该元素按钮。多个元素时，以行标签的方式显示。

### RESTCONF tools

### 2.2.2.1 RESTCONF工具介绍

RESTCONF协议提供可通过HTTP访问数据的编程接口，该数据按照YANG模型定义并存储在NETCONF协议定义的数据库中。华为控制器实现了数据库存储YANG模型数据。使用RESTCONF协议，可以对存储在数据库中的数据执行增、读、改和删 (create, read, update or delete)操作。

可以使用工具发送RESTCONF请求，如Chrome Postman plug-in 和cURL utility。 Chrome Postman plug-in也是一种图形化的工具，以下介绍Chrome Postman plug-in的使用。cURL utility是命令行工具，如果有需要，可参阅相关资料。

### 2.2.2.2 chrome Postman plug-in

使用Postman chrome应用的步骤：

1 下载并安装Postman。下载链接：https://chrome.google.com/webstore/detail/postman-rest-client/

2 打开Chrome，点击“APPlications”，选择“Postman”；

3 选择消息头：

• Authorization: 选择“basic Auth”标签, 使用admin用户名和密码，点击“Refresh Header”

• Accept: application/xml

• Content-type: application/xml

4 按照http://<controller-ip>:8181/URI输入URL；

5 选择一个REST 方法：

• GET

• PUT

• POST

• DELETE

6 点击“RAW”标签，然后选择“XML”标签，填入需要的消息体数据；

7 点击“Send”按钮。

流下发示例

为了通过Postman下发一个流，按照如下的例子设置参数即可：

注意：需要确认URL中的表ID和流ID与XML消息体中的数据匹配。

## OpenFlow

### OpenFlow简介

OpenFlow是一个开放标准，用于运行实验性的流协议，以便在网络中容易地部署新的路由或者交换功能。OpenFlow标准描述控制器接入和配置转发平面网络设备的通信协议。OpenFlow标准由控制器、转发器(支持OpenFlow协议)和OpenFlow通信协议组成。转发器和控制器通过OpenFlow协议报文相互通信；控制器通过OpenFlow报文增加、修改、删除转发器上的流配置。通过使用OpenFlow协议，控制器可以完全控制网络，如流量转发、拓扑发现、QoS等。

### OpenFlow拓扑

控制器提供了一个网络的集中逻辑视图。控制器使用LLDP消息发现连接到网络的OpenFlow交换机的拓扑和OpenFlow交换机之间的链路。拓扑管理器存储和管理控制器中的节点和链路信息。其它组件，如LLDP speaker、topology LLDP discovery和Host Tracker 共同生成拓扑数据库。以下部分描述OpenFlow拓扑发现所涉及的组件。

LLDP speacker

链路层发现协议(Link Layer Discovery Protocol ，LLDP)speacker的职责包括：

周期性发送向连接的所有交换机节点端口发送LLDP报文；

监控节点的端口状态事件。如果连接的交换机节点端口从up状态迁移到down状态，LLDP speacker不再发送报文给该节点端口；如果是从down迁移到up状态，则LLDP发送报文给该节点端口。

topology LLDP discovery

topology LLDP discovery的职责包括：

监控交换机发送给控制器的LLDP报文；

从收到的LLDP报文判定源节点、源节点端口、目的节点、目的节点端口等详细信息；

向拓扑管理器报告新链路事件；

周期性的检查链路失效并通报给拓扑管理器。链路在3个LLDP speacker周期没有收到交换机的更新信息即失效。

Host Tracker

Host Tracker的一个主要功能是发现连接到交换机端口的主机并更新拓扑信息。关于Host Tracker的更多信息，请参见2.3.6 Host Tracker。

拓扑管理器

拓扑管理器的职责包括：

监控上述模块上报的事件，存储/更新控制器数据库拓扑详细信息；

监控节点加入、删除、端口上线、下线、链路发现。拓扑管理器同样移除链路和更新拓扑数据。

通过控制器GUI获取拓扑信息

可以通过控制器GUI获取拓扑信息，参见2.2.1 控制器GUI

通过RESTCONF获取拓扑信息

可以通过发送RESTCONF请求获取拓扑信息。控制器从运维数据库获取拓扑数据并在对RESTCONF请求的响应中返回该信息。

接口访问路径：

http://<controller-ip>:8181/restconf/operational/network-topology:network-topology/

HTTP协议请求方式：

GET

参数格式：

application/xml或者application/json;charset=UTF-8

- response body参数

| 参数 | 类型 | 描述 |
| --- | --- | --- |
| topology | array[(config)topology] |  |

Topology结构

| 参数 | 类型 | 描述 |
| --- | --- | --- |
| underlay-topology | array[(config)underlay-topology] | Identifies the topology, or topologies, that this topology is dependent on. |
| node | array[(config)node] | The list of network nodes defined for the topology. |
| topology-types | object[(config)topology-types] | This container is used to identify the type, or types (as a topology can support several types simultaneously), of the topology. Topology types are the subject of several integrity constraints that an implementing server can validate in order to maintain integrity of the datastore. Topology types are indicated through separate data nodes; the set of topology types is expected to increase over time. To add support for a new topology, an augmenting module needs to augment this container with a new empty optional container to indicate the new topology type. The use of a container allows to indicate a subcategorization of topology types. The container SHALL NOT be augmented with any data nodes that serve a purpose other than identifying a particular topology type. |
| original-topology | array |  |
| topology-id | string | It is presumed that a datastore will contain many topologies. To distinguish between topologies it is vital to have UNIQUE topology identifiers. |
| link | array[(config)link] | A Network Link connects a by Local (Source) node and a Remote (Destination) Network Nodes via a set of the nodes' termination points. As it is possible to have several links between the same source and destination nodes, and as a link could potentially be re-homed between termination points, to ensure that we would always know to distinguish between links, every link is identified by a dedicated link identifier. Note that a link models a point-to-point link, not a multipoint link. Layering dependencies on links in underlay topologies are not represented as the layering information of nodes and of termination points is sufficient. |

例如，通过向controller发送如下请求可获得拓扑信息：

• Headers:

Content-type: application/xml

Accept: application/xml

Authentication: admin:admin

• URL: http://<controller-ip>:8181/restconf/operational/network-topology:network-topology/

• Method: GET

• Output:

{

"network-topology": {

"topology": [

{

"topology-id": "flow:1",

"node": [

{

"node-id": "host:00:00:01:01:01:14",

"termination-point": [

{

"tp-id": "host:00:00:01:01:01:14:openflow:4690175853636:8"

}

],

"host-tracker-service:id": "00:00:01:01:01:14",

"host-tracker-service:addresses": [

{

"id": 15,

"mac": "00:00:01:01:01:14",

"ip": "1.1.1.20",

"first-seen": 1433762053012,

"last-seen": 1433765669580

}

],

"host-tracker-service:attachment-points": [

{

"tp-id": "openflow:4690175853636:8",

"corresponding-tp": "host:00:00:01:01:01:14:openflow:4690175853636:8",

"active": true

}

]

},

{

"node-id": "openflow:152761762851",

"termination-point": [

{

"tp-id": "openflow:152761762851:313",

"opendaylight-topology-inventory:inventory-node-connector-ref": "/opendaylight-inventory:nodes/opendaylight-inventory:node/opendaylight-inventory:node[opendaylight-inventory:id='openflow:152761762851']/opendaylight-inventory:node-connector/opendaylight-inventory:node-connector[opendaylight-inventory:id='openflow:152761762851:313']"

},

{

"tp-id": "openflow:152761762851:319",

"opendaylight-topology-inventory:inventory-node-connector-ref": "/opendaylight-inventory:nodes/opendaylight-inventory:node/opendaylight-inventory:node[opendaylight-inventory:id='openflow:152761762851']/opendaylight-inventory:node-connector/opendaylight-inventory:node-connector[opendaylight-inventory:id='openflow:152761762851:319']"

},

{

"tp-id": "openflow:152761762851:315",

"opendaylight-topology-inventory:inventory-node-connector-ref": "/opendaylight-inventory:nodes/opendaylight-inventory:node/opendaylight-inventory:node[opendaylight-inventory:id='openflow:152761762851']/opendaylight-inventory:node-connector/opendaylight-inventory:node-connector[opendaylight-inventory:id='openflow:152761762851:315']"

},

{

"tp-id": "openflow:152761762851:317",

"opendaylight-topology-inventory:inventory-node-connector-ref": "/opendaylight-inventory:nodes/opendaylight-inventory:node/opendaylight-inventory:node[opendaylight-inventory:id='openflow:152761762851']/opendaylight-inventory:node-connector/opendaylight-inventory:node-connector[opendaylight-inventory:id='openflow:152761762851:317']"

}

],

"opendaylight-topology-inventory:inventory-node-ref": "/opendaylight-inventory:nodes/opendaylight-inventory:node/opendaylight-inventory:node[opendaylight-inventory:id='openflow:152761762851']"

},

{

"node-id": "host:00:00:01:01:02:14",

"termination-point": [

{

"tp-id": "host:00:00:01:01:02:14:openflow:4690175853636:10"

}

],

"host-tracker-service:id": "00:00:01:01:02:14",

"host-tracker-service:addresses": [

{

"id": 14,

"mac": "00:00:01:01:02:14",

"ip": "1.1.2.20",

"first-seen": 1433762053009,

"last-seen": 1433765669578

}

],

"host-tracker-service:attachment-points": [

{

"tp-id": "openflow:4690175853636:10",

"corresponding-tp": "host:00:00:01:01:02:14:openflow:4690175853636:10",

"active": true

}

]

},

{

"node-id": "openflow:4690175853636",

"termination-point": [

{

"tp-id": "openflow:4690175853636:8",

"opendaylight-topology-inventory:inventory-node-connector-ref": "/opendaylight-inventory:nodes/opendaylight-inventory:node/opendaylight-inventory:node[opendaylight-inventory:id='openflow:4690175853636']/opendaylight-inventory:node-connector/opendaylight-inventory:node-connector[opendaylight-inventory:id='openflow:4690175853636:8']"

},

{

"tp-id": "openflow:4690175853636:10",

"opendaylight-topology-inventory:inventory-node-connector-ref": "/opendaylight-inventory:nodes/opendaylight-inventory:node/opendaylight-inventory:node[opendaylight-inventory:id='openflow:4690175853636']/opendaylight-inventory:node-connector/opendaylight-inventory:node-connector[opendaylight-inventory:id='openflow:4690175853636:10']"

}

],

"opendaylight-topology-inventory:inventory-node-ref": "/opendaylight-inventory:nodes/opendaylight-inventory:node/opendaylight-inventory:node[opendaylight-inventory:id='openflow:4690175853636']"

}

],

"link": [

{

"link-id": "host:00:00:01:01:02:14:openflow:4690175853636:10/openflow:4690175853636:10",

"source": {

"source-node": "host:00:00:01:01:02:14",

"source-tp": "host:00:00:01:01:02:14:openflow:4690175853636:10"

},

"destination": {

"dest-tp": "openflow:4690175853636:10",

"dest-node": "openflow:4690175853636"

}

},

{

"link-id": "openflow:4690175853636:10/host:00:00:01:01:02:14:openflow:4690175853636:10",

"source": {

"source-node": "openflow:4690175853636",

"source-tp": "openflow:4690175853636:10"

},

"destination": {

"dest-tp": "host:00:00:01:01:02:14:openflow:4690175853636:10",

"dest-node": "host:00:00:01:01:02:14"

}

},

{

"link-id": "host:00:00:01:01:01:14:openflow:4690175853636:8/openflow:4690175853636:8",

"source": {

"source-node": "host:00:00:01:01:01:14",

"source-tp": "host:00:00:01:01:01:14:openflow:4690175853636:8"

},

"destination": {

"dest-tp": "openflow:4690175853636:8",

"dest-node": "openflow:4690175853636"

}

},

{

"link-id": "openflow:4690175853636:8/host:00:00:01:01:01:14:openflow:4690175853636:8",

"source": {

"source-node": "openflow:4690175853636",

"source-tp": "openflow:4690175853636:8"

},

"destination": {

"dest-tp": "host:00:00:01:01:01:14:openflow:4690175853636:8",

"dest-node": "host:00:00:01:01:01:14"

}

}

]

}

]

}

}

### OpenFlow编程概述

控制器提供编程接口对交换机进行编程。编程接口通过和南向OpenFlow模块交互，使用OpenFlow消息对交换机进行编程。

控制器主要提供RESTCONF接口对交换机进行编程，编程者可以使用任何RESTCONNF工具发送请求给控制器以便对特定交换机编程。控制器收到RESTCONF请求后，使用Java API对交换机进行编程。

控制器目前支持的节点根容器(/config/opendaylight-inventory:nodes/)下的参数见附件“controller根节点参数支持列表.xlsx”。

以下小节是使用RESTCONF接口对交换机进行编程的一些例子。

### 2.2.3.1 L2 Flow编程接口与示例

接口访问路径：

http://<controller-ip>:8181/restconf/config/opendaylight-inventory:nodes/node/{node-id}/table/{table-id}/flow/{flow-id}

HTTP协议请求方式：

PUT

参数格式：

application/xml或者application/json;charset=UTF-8

- request body参数

| 参数 | 类型 | 描述 |
| --- | --- | --- |
| table_id | integer |  |
| idle-timeout | integer |  |
| out_port | integer |  |
| match | object[(config)match] |  |
| container-name | String |  |
| flow-name | String |  |
| strict | Boolean |  |
| cookie_mask | integer |  |
| barrier | Boolean |  |
| flow-statistics | object[(config)flow-statistics] |  |
| id | String |  |
| installHw | Boolean |  |
| instructions | object[(config)instructions] |  |
| priority | integer |  |
| flags | Undefined |  |
| hard-timeout | integer |  |
| buffer_id | integer |  |
| cookie | integer |  |

以下是一个L2 flow编程的示例，该L2 flow通过匹配源MAC和目的MAC并将报文发往端口2。

通过RESTCONF client工具输入如下参数，调用RESTCONF接口实现L2 flow编程:

• Headers:

Content-type: application/xml

Accept: application/xml

• Authentication: admin:admin

• Example URL: http://localhost:8181/restconf/config/opendaylight-inventory:nodes/node/openflow:1/table/0/flow/1

• Method: PUT

• Request body:

<?xml version="1.0" encoding="utf-8"?>

<flow xmlns="urn:opendaylight:flow:inventory">

<strict>false</strict>

<instructions>

<instruction>

<order>0</order>

<apply-actions>

<action>

<order>0</order>

<output-action>

<output-node-connector>2</output-node-connector>

</output-action>

</action>

</apply-actions>

</instruction>

</instructions>

<table_id>0</table_id>

<id>1</id>

<cookie_mask>255</cookie_mask>

<match>

<ethernet-match>

<ethernet-destination>

<address>20:14:29:01:19:61</address>

</ethernet-destination>

<ethernet-source>

<address>00:00:00:00:23:ae</address>

</ethernet-source>

</ethernet-match>

</match>

<hard-timeout>12</hard-timeout>

<cookie>1</cookie>

<idle-timeout>34</idle-timeout>

<flow-name>L2-Flow</flow-name>

<priority>2</priority>

<barrier>false</barrier>

</flow>

### 2.2.3.2 L3 Flow编程接口与示例

接口访问路径：

http://<controller-ip>:8181/restconf/config/opendaylight-inventory:nodes/node/{node-id}/table/{table-id}/flow/{flow-id}

HTTP协议请求方式：

PUT

参数格式：

application/xml或者application/json;charset=UTF-8

- request body参数

| 参数 | 类型 | 描述 |
| --- | --- | --- |
| table_id | integer |  |
| idle-timeout | integer |  |
| out_port | integer |  |
| match | object[(config)match] |  |
| container-name | String |  |
| flow-name | String |  |
| strict | Boolean |  |
| cookie_mask | integer |  |
| barrier | Boolean |  |
| flow-statistics | object[(config)flow-statistics] |  |
| id | String |  |
| installHw | Boolean |  |
| instructions | object[(config)instructions] |  |
| priority | integer |  |
| flags | Undefined |  |
| hard-timeout | integer |  |
| buffer_id | integer |  |
| cookie | integer |  |

以下是一个L3 flow编程的示例,该L3 flow通过匹配IP协议报文类型及目的IP地址并将报文发往端口1。

通过RESTCONF client工具输入如下参数,调用RESTCONF接口实现L3 flow编程:

• Headers:

Content-type: application/xml

Accept: application/xml

• Authentication: admin:admin

• Example URL: http://localhost:8181/restconf/config/opendaylight-inventory:nodes/node/openflow:1/table/0/flow/1

• Method: PUT

• Request body:

<?xml version="1.0" encoding="utf-8"?>

<flow xmlns="urn:opendaylight:flow:inventory">

<hard-timeout>0</hard-timeout>

<idle-timeout>0</idle-timeout>

<priority>2</priority>

<flow-name>flow1</flow-name>

<match>

<ethernet-match>

<ethernet-type>

<type>2048</type>

</ethernet-type>

</ethernet-match>

<ipv4-destination>10.0.10.0/24</ipv4-destination>

</match>

<id>1</id>

<table_id>0</table_id>

<instructions>

<instruction>

<order>0</order>

<apply-actions>

<action>

<output-action>

<output-node-connector>1</output-node-connector>

</output-action>

<order>0</order>

</action>

</apply-actions>

</instruction>

</instructions>

</flow>

### OpenFlow路径编程

可以通过对沿路径上所有交换机进行相应的流编程实现两台连到OpenFlow网络主机的连接。流编程可以像上述章节描述的那样手工进行，也可以通过控制器支持的路径编程应用程序自动进行。

这里介绍一个华为的路径编程应用程序——Path Explorer-L2 flows。

Path Explorer应用可以自动触发在网络中交换机上下发L2流。输入源主机MAC、目的主机MAC和指定交换机信息，Path Explorer-L2 flows应用自动计算两台主机经过指定交换机的最短路径并对沿路径上的交换机下发流表。

更多关于使用Path Explorer进行二层流编程的信息，请参考《SDN大赛-华为pathexplorer-app 用户指南》。

### OpenFlow统计信息

支持OpenFlow的交换机提供统计信息的查询机制。交换机支持以下目录信息和统计信息的查询。

目录信息：

节点描述——节点描述信息，如交换机制造商、硬件版本、软件版本、序列号等；

流表特性——交换机支持的流表特性；

端口特性——交换机端口支持的特性；

统计信息：

单条流统计信息——流表中单条流的统计信息；

聚合流统计信息——表级聚合流的统计信息；

流统计信息——交换机特定流表的统计信息；

端口统计信息——节点端口的统计信息；

控制器在节点接入时获取该节点的目录和统计信息，并以3秒为周期周期性获取统计信息。控制器从接入节点获取目录和统计信息后增加到相应的运维数据区。

可以通过控制器GUI获取显示某些目录和统计信息。也可以通过RESTCONF 请求获取全部目录和统计信息。下文通过示例简要描述了通过RESTCONF请求消息从控制器获取目录和统计信息的方法。

### 2.3.5.1单条流统计信息接口与示例

接口访问路径：

http://<controller-ip>:8181/restconf/operational/opendaylight-inventory:nodes/node/{node-id}/table/{table-id}/flow/{flow-id}

HTTP协议请求方式：

GET

参数格式：

application/xml或者application/json;charset=UTF-8

- response body参数

| 参数 | 类型 | 描述 |
| --- | --- | --- |
| duration | Object |  |
| byte-count | integer |  |
| packet-count | integer |  |

示例：

向controller发送如下请求可获得单条流的统计信息：

• Headers:

Content-type: application/xml

Accept: application/xml

• Authentication: admin: admin

• Example URL: http://localhost:8181/restconf/operational/opendaylight-inventory:nodes/node/openflow:1/table/0/flow/1

• Method: GET

• Output:

{

"flow-node-inventory:flow": [

{

"id": "1",

"match": {

"ipv4-destination": "10.0.10.0/24",

"ethernet-match": {

"ethernet-type": {

"type": 2048

}

}

},

"table_id": 0,

"priority": 201,

"cookie": 0,

"idle-timeout": 0,

"instructions": {

"instruction": [

{

"order": 0,

"apply-actions": {

"action": [

{

"order": 0,

"output-action": {

"output-node-connector": "321",

"max-length": 0

}

}

]

}

}

]

},

"opendaylight-flow-statistics:flow-statistics": {

"packet-count": 0,

"byte-count": 0,

"duration": {

"second": 143,

"nanosecond": 0

}

},

"flags": "",

"hard-timeout": 0

}

]

}

### 2.3.5.2 聚合流统计信息接口与示例

接口访问路径：

http://<controller-ip>:8181/restconf/operational/opendaylight-inventory:nodes/node/{node-id}/table/{table-id}/

HTTP协议请求方式：

GET

参数格式：

application/xml或者application/json;charset=UTF-8

- response body参数

| 参数 | 类型 | 描述 |
| --- | --- | --- |
| byte-count | integer |  |
| packet-count | integer |  |
| flow-count | integer |  |

示例：

向controller发送如下请求可获得聚合流的统计信息：

• Headers:

Content-type: application/xml

Accept: application/xml

• Authentication: admin: admin

• Example URL: http://localhost:8181/restconf/operational/opendaylight-inventory:nodes/node/openflow:1/table/0

• Method: GET

• Output:

{

"opendaylight-flow-statistics:aggregate-flow-statistics": [

{

"packet-count": 784,

"byte-count": 57232,

"flow-count": 5

}

]

}

### 2.3.5.3 表统计信息接口与示例

接口访问路径：

http://<controller-ip>:8181/restconf/operational/opendaylight-inventory:nodes/node/{node-id}/table/{table-id}/

HTTP协议请求方式：

GET

参数格式：

application/xml或者application/json;charset=UTF-8

- response body参数

| 参数 | 类型 | 描述 |
| --- | --- | --- |
| packets-matched | integer |  |
| active-flows | integer |  |
| packets-looked-up | integer |  |

示例：

向controller发送如下请求可获得表的统计信息：

• Headers:

Content-type: application/xml

Accept: application/xml

• Authentication: admin: admin

• Example URL: http://localhost:8181/restconf/operational/opendaylight-inventory:nodes/node/openflow:1/table/0/

• Method: GET

• Output:

{

"opendaylight-flow-table-statistics:flow-table-statistics": [

{

"packets-looked-up": 61517517,

"active-flows": 5,

"packets-matched": 61516242

}

]

}

### 2.3.5.4 端口描述和统计信息接口与示例

接口访问路径：

http://<controller-ip>:8181/restconf/operational/opendaylight-inventory:nodes/node/{node-id}/node-connector/{node-connector-id}

HTTP协议请求方式：

GET

参数格式：

application/xml或者application/json;charset=UTF-8

- response body参数

| 参数 | 类型 | 描述 |
| --- | --- | --- |
| advertised-features | undefined | Features being advertised by the port |
| flow-capable-node-connector-statistics | object[(config)flow-capable-node-connector-statistics] |  |
| addresses | array[(config)addresses] |  |
| port-number | integer or string |  |
| supported | undefined | Features supported by the port |
| peer-features | undefined | Features advertised by peer |
| id | string | The unique identifier for the node-connector |
| maximum-speed | integer | Max port bit rate in kbps |
| name | string | Human readable name of the port |
| hardware-address | string | MAC Address of the port |
| status | enum |  |
| configuration | undefined | Bit map of OFPPC-* flags |
| state | object[(config)state] | Description of state of port |
| current-speed | integer | Current port bit rate in kbps |
| current-feature | undefined | Bit map of OFPPF-* flags |

示例：

向controller发送如下请求可获得端口描述和统计信息：

• Headers:

Content-type: application/xml

Accept: application/xml

• Authentication: admin: admin

• URL:

• Example URL: http://localhost:8181/restconf/operational/opendaylight-inventory:nodes/node/openflow:1/node-connector/openflow:1:1

• Method: GET

• Output

{

"node-connector": [

{

"id": "openflow:152761762851:321",

"address-tracker:addresses": [

{

"id": 7,

"ip": "193.2.2.2",

"first-seen": 1433387076395,

"last-seen": 1433387980534,

"mac": "00:00:c1:02:02:02"

}

],

"opendaylight-port-statistics:flow-capable-node-connector-statistics": {

"collision-count": 0,

"transmit-drops": 0,

"receive-errors": 0,

"receive-over-run-error": 0,

"receive-frame-error": 0,

"bytes": {

"transmitted": 6142717617,

"received": 0

},

"transmit-errors": 0,

"packets": {

"transmitted": 61434736,

"received": 0

},

"receive-drops": 0,

"duration": {

"second": 1433397735,

"nanosecond": 0

},

"receive-crc-error": 0

},

"flow-node-inventory:name": "GigabitEthernet1",

"flow-node-inventory:state": {

"live": true,

"blocked": false,

"link-down": false

},

"flow-node-inventory:current-speed": 0,

"flow-node-inventory:peer-features": "",

"flow-node-inventory:supported": "",

"flow-node-inventory:port-number": "321",

"flow-node-inventory:maximum-speed": 0,

"flow-node-inventory:current-feature": "",

"flow-node-inventory:hardware-address": "00:23:91:4F:84:23",

"flow-node-inventory:advertised-features": "",

"flow-node-inventory:configuration": ""

}

]

}

### 2.3.5.5 节点描述接口与示例

接口访问路径：

http://<controller-ip>:8181/restconf/operational/opendaylight-inventory:nodes/node/{node-id}

HTTP协议请求方式：

GET

参数格式：

application/xml或者application/json;charset=UTF-8

- response body参数

| 参数 | 类型 | 描述 |
| --- | --- | --- |
| serial-number | string |  |
| switch-features | object[(config)switch-features] |  |
| connected | boolean |  |
| hardware | string |  |
| pass-through | object[(config)pass-through] | When the underlying node is connected, its NETCONF context is available verbatim under this container through the mount extension |
| initial-capability | array[string] |  |
| node-connector | array[(config)node-connector] | A list of node connectors that belong this node |
| table | array[(config)table] |  |
| id | string | The unique identifier for the node |
| software | string |  |
| supported-actions | object[(config)supported-actions] |  |
| manufacturer | string |  |
| description | string |  |
| supported-instructions | object[(config)supported-instructions] |  |
| supported-match-types | object[(config)supported-match-types] |  |
| ip-address | undefined | IP address of a flow capable node |
| current-capability | array[string] |  |

示例：

向controller发送如下请求可获得节点描述信息：

• Headers:

Content-type: application/xml

Accept: application/xml

• Authentication: admin: admin

• Example URL: http://localhost:8181/restconf/operational/opendaylight-inventory:nodes/node/openflow:1/

Method: GET

• Output:

{

"node": [

{

"id": "openflow:152761762851",

"flow-node-inventory:serial-number": "0231C001",

"flow-node-inventory:software": "S6710 V200R006C01",

"flow-node-inventory:manufacturer": "Huawei Technologies Co., Ltd",

"flow-node-inventory:description": "Designed by Huawei in China",

"flow-node-inventory:hardware": "OpenFlow1.3 Switch Quidway S6710-52Q-EI",

"flow-node-inventory:ip-address": "3.3.3.3",

"flow-node-inventory:switch-features": {

"capabilities": [

"flow-node-inventory:flow-feature-capability-flow-stats",

"flow-node-inventory:flow-feature-capability-port-stats",

"flow-node-inventory:flow-feature-capability-table-stats",

"flow-node-inventory:flow-feature-capability-queue-stats",

"flow-node-inventory:flow-feature-capability-port-blocked",

"flow-node-inventory:flow-feature-capability-group-stats"

],

"max_buffers": 1024,

"max_tables": 9

},

"flow-node-inventory:meter": [

{

"meter-id": 1,

"meter-band-headers": {

"meter-band-header": [

{

"band-id": 0,

"band-rate": 50,

"band-burst-size": 500,

"dscp-remark-burst-size": 500,

"dscp-remark-rate": 50,

"meter-band-types": {

"flags": "ofpmbt-dscp-remark"

}

}

]

},

"opendaylight-meter-statistics:meter-statistics": {

"meter-band-stats": {

"band-stat": [

{

"band-id": 0,

"byte-band-count": 0,

"packet-band-count": 0

}

]

},

"duration": {

"nanosecond": 0,

"second": 357

},

"byte-in-count": 0,

"packet-in-count": 0,

"flow-count": 0,

"meter-id": 1

},

"opendaylight-meter-statistics:meter-config-stats": {

"meter-band-headers": {

"meter-band-header": [

{

"band-id": 0,

"band-rate": 50,

"dscp-remark-rate": 50,

"dscp-remark-burst-size": 500,

"band-burst-size": 500,

"meter-band-types": {

"flags": "ofpmbt-dscp-remark"

}

}

]

},

"meter-id": 1

}

}

],

"flow-node-inventory:table": [

{

"id": 0,

"opendaylight-flow-statistics:aggregate-flow-statistics": {

"packet-count": 1036,

"byte-count": 75628,

"flow-count": 5

},

"opendaylight-flow-table-statistics:flow-table-statistics": {

"packets-looked-up": 61517770,

"active-flows": 5,

"packets-matched": 61516495

}

},

{

"id": 1,

"opendaylight-flow-statistics:aggregate-flow-statistics": {

"packet-count": 0,

"byte-count": 0,

"flow-count": 0

},

"opendaylight-flow-table-statistics:flow-table-statistics": {

"packets-looked-up": 81486,

"active-flows": 0,

"packets-matched": 81486

}

},

{

"id": 2,

"opendaylight-flow-statistics:aggregate-flow-statistics": {

"packet-count": 0,

"byte-count": 0,

"flow-count": 0

},

"opendaylight-flow-table-statistics:flow-table-statistics": {

"packets-looked-up": 81092,

"active-flows": 0,

"packets-matched": 81092

}

},

{

"id": 3,

"opendaylight-flow-statistics:aggregate-flow-statistics": {

"packet-count": 0,

"byte-count": 0,

"flow-count": 0

},

"opendaylight-flow-table-statistics:flow-table-statistics": {

"packets-looked-up": 81085,

"active-flows": 0,

"packets-matched": 81085

}

},

{

"id": 8,

"opendaylight-flow-statistics:aggregate-flow-statistics": {

"packet-count": 0,

"byte-count": 0,

"flow-count": 0

},

"opendaylight-flow-table-statistics:flow-table-statistics": {

"packets-looked-up": 81085,

"active-flows": 0,

"packets-matched": 81085

}

},

{

"id": 4,

"opendaylight-flow-statistics:aggregate-flow-statistics": {

"packet-count": 0,

"byte-count": 0,

"flow-count": 0

},

"opendaylight-flow-table-statistics:flow-table-statistics": {

"packets-looked-up": 81085,

"active-flows": 0,

"packets-matched": 81085

}

},

{

"id": 5,

"opendaylight-flow-statistics:aggregate-flow-statistics": {

"packet-count": 0,

"byte-count": 0,

"flow-count": 0

},

"opendaylight-flow-table-statistics:flow-table-statistics": {

"packets-looked-up": 81085,

"active-flows": 0,

"packets-matched": 81085

}

},

{

"id": 6,

"opendaylight-flow-statistics:aggregate-flow-statistics": {

"packet-count": 0,

"byte-count": 0,

"flow-count": 0

},

"opendaylight-flow-table-statistics:flow-table-statistics": {

"packets-looked-up": 81085,

"active-flows": 0,

"packets-matched": 81085

}

},

{

"id": 7,

"opendaylight-flow-statistics:aggregate-flow-statistics": {

"packet-count": 0,

"byte-count": 0,

"flow-count": 0

},

"opendaylight-flow-table-statistics:flow-table-statistics": {

"packets-looked-up": 81085,

"active-flows": 0,

"packets-matched": 81085

}

}

],

"opendaylight-meter-statistics:meter-features": {

"max_meter": 256,

"meter-band-supported": [

"opendaylight-meter-types:meter-band-dscp-remark",

"opendaylight-meter-types:meter-band-drop"

],

"max_bands": 16,

"max_color": 8,

"meter-capabilities-supported": [

"opendaylight-meter-types:meter-kbps"

]

},

"opendaylight-group-statistics:group-features": {

"actions": [

159383553

],

"max-groups": [

4294967040

],

"group-types-supported": [

"opendaylight-group-types:group-ff",

"opendaylight-group-types:group-all",

"opendaylight-group-types:group-select",

"opendaylight-group-types:group-indirect"

],

"group-capabilities-supported": [

"opendaylight-group-types:chaining",

"opendaylight-group-types:chaining-checks",

"opendaylight-group-types:select-liveness",

"opendaylight-group-types:select-weight"

]

},

"node-connector": [

{

"id": "openflow:152761762851:321",

"address-tracker:addresses": [

{

"id": 7,

"ip": "193.2.2.2",

"first-seen": 1433387076395,

"last-seen": 1433387980534,

"mac": "00:00:c1:02:02:02"

}

],

"opendaylight-port-statistics:flow-capable-node-connector-statistics": {

"collision-count": 0,

"transmit-drops": 0,

"receive-errors": 0,

"receive-over-run-error": 0,

"receive-frame-error": 0,

"bytes": {

"transmitted": 6142729926,

"received": 0

},

"transmit-errors": 0,

"packets": {

"transmitted": 61434893,

"received": 0

},

"receive-drops": 0,

"duration": {

"second": 1433398000,

"nanosecond": 0

},

"receive-crc-error": 0

},

"flow-node-inventory:name": "GigabitEthernet1",

"flow-node-inventory:state": {

"live": true,

"blocked": false,

"link-down": false

},

"flow-node-inventory:current-speed": 0,

"flow-node-inventory:peer-features": "",

"flow-node-inventory:supported": "",

"flow-node-inventory:port-number": "321",

"flow-node-inventory:maximum-speed": 0,

"flow-node-inventory:current-feature": "",

"flow-node-inventory:hardware-address": "00:23:91:4F:84:23",

"flow-node-inventory:advertised-features": "",

"flow-node-inventory:configuration": ""

},

{

"id": "openflow:152761762851:322",

"address-tracker:addresses": [

{

"id": 6,

"ip": "193.2.1.2",

"first-seen": 1433387076394,

"last-seen": 1433387980534,

"mac": "00:00:c1:02:01:02"

}

],

"opendaylight-port-statistics:flow-capable-node-connector-statistics": {

"collision-count": 0,

"transmit-drops": 0,

"receive-errors": 0,

"receive-over-run-error": 0,

"receive-frame-error": 0,

"bytes": {

"transmitted": 3234510,

"received": 0

},

"transmit-errors": 0,

"packets": {

"transmitted": 45169,

"received": 0

},

"receive-drops": 0,

"duration": {

"second": 1433398000,

"nanosecond": 0

},

"receive-crc-error": 0

},

"flow-node-inventory:name": "GigabitEthernet1",

"flow-node-inventory:state": {

"live": true,

"blocked": false,

"link-down": false

},

"flow-node-inventory:current-speed": 0,

"flow-node-inventory:peer-features": "",

"flow-node-inventory:supported": "",

"flow-node-inventory:port-number": "322",

"flow-node-inventory:maximum-speed": 0,

"flow-node-inventory:current-feature": "",

"flow-node-inventory:hardware-address": "00:23:91:4F:84:23",

"flow-node-inventory:advertised-features": "",

"flow-node-inventory:configuration": ""

}

]

}

]

}

### Host Tracker

Host Tracker是OpenDaylight(ODL) 2层交换机应用的一个组件。2层交换机有如下几个与Host Tracker相关的组件：

Packet Handler：该组件将报文分配到流，收集报文信息并将报文发往目的地；

环回消除：相互连接的交换机由于冗余的需要会形成交换环路。报文在环路上不停往返传输导致网络资源无限浪费。环路消除组件利用LLDP报文学习拓扑信息，然后使用该信息避免在环路上转发报文。

ARP处理器：2层交换机使用ARP 处理器处理泛洪并将地址解析协议(Address Resolution Protocol ,ARP)报文发给控制器。当到达交换机的报文不能匹配流表或者ARP报文，该报文被发往控制器。另一个选择是将该报文在网络中泛洪。

Address Tracker：该组件通过ARP请求和响应报文，跟踪IP地址到MAC地址；

Host Tracker：该组件监控主机在网络中的位置(将主机的MAC地址作为主标识)并将主机加入到拓扑。Host Tracker创建一个代表主机的节点并将它关联到相应的交换机。

这些组件在实现上并不是相互独立的。例如，环回消除依赖Packet Handler的实现；而Host Tracker需要基于Address Tracker实现，进而依赖Packet Handler。

Host Tracker触发流量以发现主机和拓扑及网络泛洪。Host Tracker由此获得主机在网络中的位置并收集到主机的业务流信息。

# 缩略语

| 术语/缩略语 | 描述 |
| --- | --- |
| SDN | Software Difined Network |
| GUI | Graphical User Interface |
| XML | eXtensible Markup Language |
| JSON | JavaScript Object Notation |
| LLDP | Link Layer Discovery Protocol |
| API | Application Programming Interface |
| TLS | Transport Layer Security |
| PKI | Private Key Infrastructure |
| ODL | OpenDaylight |